EE334 	Name: _________________________________

# Computer Architecture	Homework Assignment 6	March 31 2010

### Problem 1.

A program is run on a load and store processor that has a clock cycle time of 0.2nsec. The instruction mix is given below.

| Instruction type | Executed instructions | Multi-cycle CPIi | Pipeline CPIi |
| --- | --- | --- | --- |
| ALU | 9.3X108 | 4 | 1 |
| Load | 8.1X108 | 5 | 1 |
| Store | 4.3 X108 | 4 | 1 |
| FP inst. | 1.2 X108 | 6.8 | 4.8 |
| Branch | 3.4 X108 | 3 | 2 |

Please determine:

- The CPI for each implementation.

- What implementation is faster and by how much.

### Problem 2.  Control and Data Hazards

Clara, now a senior engineer, at C3 (Creative Computer Corporation) is working on a new pipelined processor. She has a benchmark that has the following characteristics:

33%	ALU instructions,

27%	load instructions (15% of loads are followed by instructions that use the data being loaded)

16%	store instructions

24%	branch instructions (58% of these branches are taken).

This processor’s CPIint is 1 (when there are no hazards). Please answer the following questions:

- Considering control and data hazards, please compute the overall CPI. Assume that all branch instructions cause hazards (the penalty is 1 clock cycle; actually, a NO-OP instruction is inserted after every branch).

- Clara has an idea, she would move some instructions (in order to remove some NO-OPs) without causing any side effects on the program execution. Remember, the instruction after the branch is always executed by this machine.  She could replace some of the NO-OPs as follows: 11% with an instruction from the before the branch and 9% with an instruction from the target. Please compute the new CPI.

##### BRANCH

:

Problem 3.

A benchmark in a five-stage pipelined processor has the following characteristics:

26%	ALU instructions,

23%	load instructions

14% of the loads are followed by instructions that use the data being loaded.

25% of these loads are followed by stores that write in memory the data being loaded.

14%	store instructions

19%	floating point instructions, and

18%	branch instructions (54% of these branches are taken).

This processor’s CPIint and CPIFP are equal to 1 and 3.1, respectively (when there are no hazards). Please answer the following questions:

- Assuming that all the branch instructions cause hazards (the penalty is 1 clock cycle), please compute the overall CPI.

- If the branch delay slot is scheduled using the three strategies and NO-OP as follows:

| ## Delay Slot | NO-OP | Fall through | Target | Before |
| --- | --- | --- | --- | --- |
| % | 44% | 11% | 23% | 22% |