EE334 Computer Architecture	Name: _________________________________

# Homework Assignment 6		Due: March 9, 2011, 9:10am

A program is run on a load and store processor that has a clock cycle time of 0.34nsec. The number of instructions that are executed is 1.87X109 and the instruction mix is given below.

| Instruction type | Executed instructions | Multi-cycle CPIi | Pipeline CPIi |
| --- | --- | --- | --- |
| ALU | 8.7X108 | 4 | 1 |
| Load | 3.9X108 | 5 | 1 |
| Store | 1.2 X108 | 4 | 1 |
| FP inst. | 1.5 X108 | 6.8 | 4.2 |
| Branch | 3.4 X108 | 3 | 2 |

Please determine:

- The CPI for each implementation.

- What implementation is faster and by how much.