Homework 5	Due: Friday February 25, 2011

For this problem you are to use the code on the right that implements a loop. Let us assume that the code is loaded onto the memory starting at location 0x08000 (0x means that the number that follows is an hexadecimal).

a) Please determine the format of each instruction, its location in memory, and its code. Use the table below.

ori 	$a3, $0, 6

lw 	$a1, 4, ($t0)

sum:	beq 	$a3, $0, sum_xt

add	$5, $5, $7

addi	$7, $7, -1

j 	sum

sum_xt:	lui 	$4, 4097

| Instruction | Format | Address (Hex) | Instruction code (decimal notation) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ori $a3, $0, 6 |  |  |  |  |  |  |  |  |
| lw $a1, 4($t0) |  |  |  |  |  |  |  |  |
| beq $a3,$0,sum_xt |  |  |  |  |  |  |  |  |
| add $5, $5, $7 |  |  |  |  |  |  |  |  |
| addi $7, $7, -1 |  |  |  |  |  |  |  |  |
| j sum |  |  |  |  |  |  |  |  |
| lui $4, 4097 |  |  |  |  |  |  |  |  |

b) Instructions from memory location 0x08000 to 0x08010 are executed; ie: the initial PC is 0x08000 and the last instruction that is executed is at location 0x08010. The initial conditions of the registers and memory are shown below.

## Initial setting

Registers								Memory

0: 0x0000	 8: 0x08808	16: 0x0000	24: 0x08846		0x08800: 0x0043

1: 0x0CA1	 9: 0x08820	17: 0x0000	25: 0x08855		0x08804: 0x0056

2: 0x0675	10: 0x08830	18: 0x0000	26: 0x0CC3		0x08808: 0x0033

3: 0x0033	11: 0x08840	19: 0x0001	27: 0x0B45		0x0880C: 0x0045

4: 0x0011 	12: 0x08730	20: 0x0008	28: 0x0DEF		0x08800: 0x0089

5: 0x0056	13: 0x04489	21: 0x0032	29: 0x0F000		0x08810: 0x08086

6: 0x0FFF	14: 0x05589	22: 0x0064	30: 0x0E330		0x08814: 0x02000

7: 0x0013	15: 0x03366	23: 0x0095	31: 0x04000		0x08818: 0x02001

Please determine the condition of the registers and memory after the instruction at location 0x08010 is executed. Please include only new values in the table below.

## Setting after execution

Registers								Memory

0: 0x0000 	 8: 0x0		16: 0x0		24: 0x0			0x08800: 0x0

1: 0x0		 9: 0x0		17: 0x0		25: 0x0			0x08804: 0x0

2: 0x0 		10: 0x0		18: 0x0		26: 0x0			0x08808: 0x0

3: 0x0		11: 0x0		19: 0x0		27: 0x0			0x0880C: 0x0

4: 0x0 		12: 0x0		20: 0x0		28: 0x0			0x08800: 0x0

5: 0x0		13: 0x0		21: 0x0		29: 0x0			0x08810: 0x0

6: 0x0		14: 0x0		22: 0x0		30: 0x0			0x08814: 0x0

7: 0x0		15: 0x0		23: 0x0		31: 0x0			0x08818: 0x0